Corridor presence sensor for ceiling installation, light and HVAC, 30x4m (compact passage)

Theben HTS 360° presence sensor for ceiling installation, fully automatic/semi-automatic options, sensor or switch control, blended light measurement ~ 10...1500 lux, 2 relays for light and presence, square detection area 30x4.5 m at installation height of 3 m, light relay: 1 NO contact 6 A 1400 VA, delay 10 s...20 min auto-learning / short pulse, presence relay: 1 floating NO contact, 250 V AC, 50 W/50 VA, response delay 0 s...10 min / room monitoring, release delay 10 s...120 min, remote control, front plate insulation material white 102.5x102.5x44 mm, mounting plate 70x70 mm, installation depth 40 mm.

Deliver, assemble, connect up and set up

Make: Theben

Type: compact passage

Order No.: 2010090

Unit: ST

Accessories:

# QuickSet plus

Service remote control

Order No.: 9070532

Clik

User remote control

Order No.: 9070515

# AP frame compact, white

Order No.: 9070514