EIB/KNX thermostat for individual rooms, continuous control or 2 point control (RAM 713 EIB/KNX)

EIB/KNX thermostat for individual room control; installation housing 84 x 80 x 27 mm; colour: white; continuous individual room control with integral bus coupler; setting range 10-28°C; applications: Heating via radiator or underfloor heating; 2-step heating with basic and additional stages (underfloor and radiator heating); heating and cooling, e.g. radiator and cooling surface. Temperature control possible via external floor sensor (5 - 46°C); manual switch for presence or for selecting operating modes: comfort, standby, temperature reduction at night, frost protection 3 binary inputs for conventional keys with the following functions: switching, dimming, blinds Inputs can also be used for external temperature sensors, window contact or presence sensor.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: RAM 713 EIB/KNX

Order No.: 7139201

Unit: ST