# Tender text for presence detectors

# LON presence detectors for ceiling installation PlanoCentro UWH PCLON / UBK PCLON / USR PCLON

Passive infrared presence detector for flush-mount/surface-mount installation; flat design, with metal installation frame for screw attachment in flush-mount/surface-mount socket for concrete ceiling, plaster ceiling or wooden ceiling; square 360° detection area, walking 100 m2 (10 x 10 m), seated 64 m2 (8 x 8 m) at 3.5 m installation height;

Automatic presence and brightness-dependent control for lighting and HVAC;

Switching or constant light control; choice of fully or semi-automatic operation;

Mixed light measurement 3-fold approx. 10 – 2000 Lux;

Topology LON FTT, Operating voltage 24 V AC/DC;

2 independent Constant Light Controller #3050 for control of two light groups, switching or constant light control with stand-by functionality, brightness set point value with configurable network variable;

3 x Occupancy Controller #3071 for control of the constant light controller or use as presence channels, switch-off delay 10 s – 100 min, switch-on delay 0 s – 30 min;

3 x Light Sensor #1010, can be calibrated individually, selectable light measurement;

Occupancy Sensor #1060;

Scene Panel #3250 can be used with the SendoClic user remote control;

Scene Controller #3251;

Parallel switching of several presence detectors without logic modules, plug-in for convenient configuration of parameters and functionality, remotely configurable and reading of the set parameters; white cover (UWH), black (UBK), silver (USR) 123 x 123 x 5 mm, can be changed according to design, colour and material; protection rating IP 40 in fitted state

Manufacturer: thebenHTS

Type: PlanoCentro U.. PCLON

Order No.:	206 9 202 (PlanoCentro UWH PCLON, white, similar to RAL 9010)

206 9 203 (PlanoCentro UBK PCLON, black, similar to RAL 9005)

206 9 204 (PlanoCentro USR PCLON, silver, similar to RAL 9006)

Unit: ST

### Accessories

### SendoPro 868-A management remote control

Optional for the installer for start-up or adjustment to usage conditions.

Text-based operator guidance in the display. Transmission of individual settings or complete configuration sets to the detector possible. Suitable for all remote controlled thebenHTS presence detectors.

Order No.: 9070675

### SendoClic user remote control

Optional

Order No.: 9070690

### Flush-mounted socket, 115 x 115 x 100 mm

Agro/Kaiser flush-mounted junction box 9908.01

Order No.: 9070689