# Tender text

# LON presence detector; ceiling installation PresenceLight 360 PLLON WH / BK / SR

Passive infrared (PIR) presence detector for ceiling installation; square 360° detection area max. moving range 49 m2 (7x7 m) stationary 20 m2 (4.5x4.5 m) at 3 m installation height; fully automatic light control; can be operated semi or fully automatically; manual push button control; mixed light measurement 5-2000 lx;

Topology LON FTT, Operating voltage 24 V AC/DC;

2 independent Constant Light Controller #3050 for control of two light groups, switching or constant light control with stand-by functionality, brightness set point value with configurable network variable;

3 x Occupancy Controller #3071 for control of the constant light controller or use as presence channels, switch-off delay 10 s – 100 min, switch-on delay 0 s – 30 min;

Light Sensor #1010, can be calibrated;

Occupancy Sensor #1060;

Scene Panel #3250 can be used with the SendoClic user remote control;

Scene Controller #3251;

Parallel switching of several presence detectors without logic modules, plug-in for convenient Configuration of parameters and functionality, remotely configurable and reading of the set parameters;

protection class IP40; integrated bus coupling unit and power supply via bus; front plate insulation material white 86x86x27 mm, mounting plate 70x70 mm, installation depth 38 mm.

Manufacturer: thebenHTS

Type: PresenceLight 360 PLLON

Order No.:	200 9 100 (PresenceLight 360 PLLON, white, similar to RAL 9010)

200 9 101 (PresenceLight 360 PLLON, black, similar to RAL 9005)

200 9 102 (PresenceLight 360 PLLON, silver, similar to RAL 9006)

Unit: ST

Accessories

“SendoPro 868-A” management remote control

Optional for the installer for start-up or adjustment to usage conditions.

Text-based operator guidance in the display. Transmission of individual settings or complete configuration sets to the detector possible. Suitable for all remote controlled thebenHTS presence detectors.

Order No.: 9070675

“SendoClic” user remote control

Optional

Order No.: 9070690

AP frame Presence Light, white

Order No.: 9070513

Protective cage QuickSafe

Order No.: 9070531