Mini programmable controller DC, 6 analogue/digital inputs, 4 outputs

Rail mounted mini programmable controller; quick fastening or wall installation; width 72 mm; 34 different function blocks are integrated; each of which can be used up to 200 times; programming directly on the device or via the easy-to-use software; 3 level password protection; remote control / remote monitoring possible via modem or GSM modem; transmission of SMS messages, e-mail or fax can be implemented; upgradeable with modules containing 4 outputs or inputs. Function blocks: logical functions, Boolean, set reset, time delay, pulse shaper, edge evaluation, clock generator, current surge relay, elapsed-time counter, counter, signal converter (gain), display (4 lines of 12 characters each for data or comments), range compare function, Schmitt trigger (two-step controller), PWM, frequency-dependent switching, GSM/SMS, random-check generator, SHIFT function, addition, subtraction, multiplication, division, arithmetic function equation. LCD display with operator guidance in plain text. Input and output switch status indication and MMI function. Operating voltage 24V~/ 50 Hz; 6 analogue/digital inputs selectable; 4 relay outputs with 8A/250V~ each.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: PHARAO-II 11 (DC)

Order No.: 5750211

Unit: ST